ZMON V2.00 DOCUMENTATION

MONITOR FUNCTION CALLS:

These functions are executed by placing the arguments into the

proper registers and executing a Restart 20H, or by calling location

F806H.

Arguments:

A= Function #

BC= Arg #1

DE= Arg #2

HL= Arg #3

E:HL= Banked memory ptr

Return Values:

A= Byte

HL= Word

BC, DE= Additional values

Function descriptions:

RETURN MONITOR ROM VERSION #

ENTRY-	A= 0

EXIT -	HL= BINARY VERSION # IN HUNDREDTHS (X.XX)

***  MEMORY MANAGEMENT  ***

RETURN BANKED MEMORY DATA

ENTRY-	A= 1

E:HL= PTR TO DATA LOCATION

EXIT -	A= DATA FROM LOCATION

STORE BANKED MEMORY DATA

ENTRY-	A= 2

E:HL= DATA PTR

C= DATA TO STORE INTO LOCATION

SELECT MEMORY BANK FOR EXECUTION

ENTRY-	A= 3

C= BANK #

SELECT MEMORY BANKS FOR MOVE

ENTRY-	A= 4

B= SOURCE BANK #

C= DESTINATION BANK #

BANK BLOCK MOVE

ENTRY-	A= 5

HL= SOURCE PTR

DE= DESTINATION PTR

BC= BYTE COUNT

EXIT -	HL= LAST SOURCE BYTE PTR + 1

DE= LAST DESTINATION BYTE PTR + 1

BC= 0

EXECUTE BANK

ENTRY-	A= 6

E':HL'= PTR TO EXECUTION CODE

*** Character I/O Device Functions ***

RETURN CHAR DEVICE VERSION #

ENTRY-	A= 7

D= 0

EXIT -	HL= BINARY VERSION # IN HUNDREDTHS (X.XX)

RETURN CHAR DEVICE PARAMETERS

ENTRY-	A= 7

D= 1

B= DEVICE # {0..15}, -1, -2}

E:HL= PTR TO LOCATION TO PLACE DATA

EXIT -	A= 0: DEVICE NOT PRESENT

/0: BUFFER CONTENTS (BY OFFSET)

(0)= 6 CHAR ASCII LOGICAL DEVICE NAME

(6)= MODE BITS:

D0= 1: INPUT DEVICE

D1= 1: OUTPUT DEVICE

D2= 1: SELECTABLE BAUD RATE

D3= 1: XON PROTOCOL OK

D4= 1: XON/XOFF ENABLED

(7)= BAUD RATE INDEX INITIALIZED TO

EXECUTE SELF TEST IF APPLICABLE

ENTRY-	A= 7

D= 2

B= DEVICE # {0..15}, -1, -2}

EXIT -	A= 0: OK, /0: ERROR CODE

SET STANDARD CONSOLE DEVICE

ENTRY-	A= 7

D= 3

B= DEVICE # TO SET TO {0..15}, -1, -2}

EXIT -	A= 0..15: PREVIOUS DEVICE

-1: ILLEGAL DEVICE

SET STANDARD LIST DEVICE

ENTRY-	A= 7

D= 4

B= DEVICE # TO SET TO {0..15}, -1, -2}

EXIT -	A= 0..15: PREVIOUS DEVICE

-1: ILLEGAL DEVICE

INITIALIZE CHAR DEVICE

ENTRY-	A= 7

D= 5

B= DEVICE # {0..15}, -1, -2}

SET DEVICE BAUD RATE

ENTRY-	A= 7

D= 6

B= DEVICE # {0..15}, -1, -2}

C= BAUD RATE INDEX {0: NONE, 1..15}

EXIT -	A= 0..15: PREVIOUS BAUD RATE INDEX

FFH: ILLEGAL BAUD RATE

*** Disk I/O Driver Functions ***

RETURN DISK DRIVER VERSION #

ENTRY-	A= 8

D= 0

EXIT -	HL= BINARY VERSION # IN HUNDREDTHS (X.XX)

BOOT DISK

ENTRY-	A= 8

D= 1

BC= PHYSICAL DRIVE #

BITS 2..0= ROM #

3= 0

7..4= DRIVE #

15..8= CONTROLLER #

EXIT -	ERROR, WILL NOT RETURN IF SUCCESSFUL

TEST IOPB.PBSTATO FOR ERROR AND DUMP IOPB IF TRUE

ENTRY-	A= 8

D= 2

E:HL= IOPB PTR

EXIT -	AF= 0: NO ERROR, /0: ERROR CODE

DISPLAY IOPB ERROR DATA

ENTRY-	A= 8

D= 3

E:HL= IOPB PTR

EXIT -	AF= 0: NO ERROR, /0: ERROR CODE

EXECUTE IOPB

ENTRY-	A= 8

D= 4

E:HL= IOPB PTR

EXIT -	AF= 0: NO ERRORS, /0: ERROR CODE

*** Miscellaneous I/O Driver Functions ***

RETURN MISCELLANEOUS DRIVER VERSION

ENTRY-	A= 9

D= 0

EXIT -	HL= BINARY VERSION # IN HUNDREDTHS (X.XX)

READ CLOCK BUFFER

ENTRY-	A= 9

D= 1

E:HL= PTR TO 13 BYTE BUFFER TO PLACE DATA

WRITE CLOCK BUFFER

ENTRY-	A= 9

D= 2

E:HL= PTR TO 13 BYTE BUFFER TO SET CLOCK FROM

RETURN CLOCK CALENDAR DATA

ENTRY-	A= 9

D= 3

EXIT -	A= DAY

B= MONTH

C= YEAR

L= DAY OF WEEK (0= SUNDAY, 1= MONDAY..)

RETURN CLOCK TIME DATA

ENTRY-	A= 9

D= 4

EXIT -	A= SECONDS

B= MINUTES

C= HOURS

RETURN CLOCK REGISTER DATA

ENTRY-	A= 9

D= 5

L= REGISTER #

EXIT -	A= DATA FROM CLOCK REGISTER

WAIT FOR CLOCK TO CHANGE. DOES NOT RETURN UNTIL CLOCK SECONDS CHANGE.

NOTE: INSURE BOARD IS IN SYSTEM WITH FUNCTION #7 BEFORE

INVOKING, OTHERWISE, THE SYSTEM WILL 'HANG'!

ENTRY-	A= 9

D= 6

RETURN CLOCK BOARD PRESENT STATUS

ENTRY-	A= 9

D= 7

EXIT -	A= 0: NOT IN SYSTEM, /0: BOARD IN SYSTEM

STOP CLOCK

ENTRY-	A= 9

D= 8

START CLOCK

ENTRY-	A= 9

D= 9

#####################################################################################